BIG YOUTH

Technical & Artist Rider

This Technical & Artist Rider forms part of, and is incorporated by reference into, the Big Youth Booking Agreement. All requirements are the responsibility of the Purchaser/Promoter unless agreed otherwise in writing.

## Select Engagement Tier

Promoter/booking agent: tick the tier for this engagement. Requirements below scale to the selected tier.

☐ Tier A — Calendar / Private Engagement (symposium, speaking, cultural event, meet & greet). No performance. Party 3–5.

☐ Tier B — Club / Selector Set (200–700 capacity). 30–45 min to instrumentals with accompanying DJ. Party 3–7.

☐ Tier C — Concert / Festival (1,000–30,000). Live band. Party 9–21.

☐ Tier C — 5-piece band

☐ Tier C — 17-piece band

☐ Add touring DJ

☐ Full Tour (3–9 weeks; mixed tiers — see Booking Agreement)

## Stage & Power

- Level, weatherproof (if outdoor) stage. Min. 20' × 16' (Tier C 5-pc); 32' × 24' (Tier C 17-pc); DJ riser 8' × 8' (Tier B).

- Clean, grounded power: separate audio and lighting circuits; distro to FOH and stage.

- Roof/cover for all outdoor performances protecting artists and equipment.

## FOH / PA System

- Professional line-array PA (d&b, L-Acoustics, Meyer or equiv.) providing even, undistorted 105 dB(A) at FOH.

- FOH console: digital (DiGiCo, Avid S6L, Yamaha CL/QL or equiv.) with min. 32 inputs (Tier C).

- Experienced FOH engineer provided by Purchaser; Artist may travel own engineer (Tier C) — advise at advance.

## Monitors

- Tier B: 2 wedge mixes or stereo IEM for Big Youth + DJ foldback.

- Tier C: min. 6 monitor mixes (5-pc) / 12 mixes (17-pc); IEM preferred for Big Youth.

- Dedicated monitor engineer and console for Tier C.

## Microphones & Input List

Indicative list; final patch confirmed at advance per selected tier.

| Ch | Source | Provision |
| --- | --- | --- |
| 1–2 | Lead Vocal — Big Youth | Wireless handheld (Shure UR/Axient or equiv.) + wired backup |
| 3–4 | Guest / MC Vocals | Wired SM58 (Tier C) |
| 5–6 | DJ / Playback L–R | Stereo DI from DJ mixer (Tier B & C+DJ) |
| 7–12 | Keys / Bass / Guitar DIs | Active DIs (Tier C) |
| 13–24 | Drums / Percussion | Full kit + Nyabinghi hand-drums mic'd (Tier C 17-pc) |
| 25–32 | Horns / Backing Vocals | Condenser / clip mics (Tier C 17-pc) |

## Backline

Tier B (to tracks + DJ):

- 2× professional CDJs (Pioneer CDJ-3000) + DJM-A9/900 mixer, on riser, booth monitor.

Tier C (live band — provided by Purchaser unless carried by Artist; confirm at advance):

- Drum kit (5/6-piece), bass rig, 2× guitar amps, keyboards (88-key weighted + organ), percussion/Nyabinghi drums.

- 17-piece adds: full horn section stands + mics, additional keys, backing-vocal mics, expanded percussion.

## Lighting

- Full stage wash with control; front spot for Big Youth.

- Rasta wash option (red / gold / green) available on request.

## DJ Requirements (Tier B & Tier C + DJ)

- Table/riser for DJ, dedicated stereo channel to FOH, booth monitor, power at booth.

## Stage Plot & Patch

Stage plot and full input list to be attached per selected tier at advance (references on file: Kabaka Pyramid, Sean Paul, Third World, Jo Mersa).

## Advance & Schedule

- Production advance to be completed no later than 14 days prior to performance.

- Soundcheck: min. 60 min (Tier B) / 90–120 min (Tier C), stage clear, PA tuned, before doors.

Big Youth Music Ltd.  · Management: Rud3 Music Management · Partnered with & Powered by Reggae Music Global Inc.

bookings@bigyouthmusic.com · +1 305 783 7282 · +1 786 614 6908